- S9500-6QF-2HQF，100G核心交换机

- 产品概述

S9500-6QF-2HQF交换机是公司一款高性能高速率产品，支持40G下行和100G上行核心路由三层交换机产品。既可满足下一代企业网、数据中心和城域网对于高密度高带宽的网络 应用需求，也可满足电信网络运营商或园区以太网汇聚层或接入层应用需求，亦可作为数据 中心接入层、中小企业核心层，为数据中心提供丰富的服务器接入方案。

- 产品外观

| S9500-6QF-2HQF 6*40G QSFP端口 2*100G QSFP28 端口 |  |
| --- | --- |

- 产品特点

- 采用运营商级、高性能以太网核心交换芯片

交换机采用国产自研高性能以太网核心交换芯片，可满足城域网、企业网、数据中心、超融合等应用的需求。

- 弹性端口形态、内置配置模板、清晰的端口状态指示

原生40G/100G端口形态，通过端口分拆可支持10G/25G模式。分拆后的逻辑端口有独立的端口状态指示灯，可清晰显示每个拆分端口的收发包工作状态方便运维。预制多种配置模板，一键切换端口模式：6*40G+2*100G/24*10G+2*100G/24*10G+8*25G/32*10G；

- 绿色节能的系统设计

采用智能的风扇调速方案，支持实时功耗检测，为客户降低运维成本，打造绿色节能网络。

- 可灵活配置的转发表项满足不同业务部署场景

灵活的表项管理技术FTM，支持根据不同的应用场景需求，调整各类硬件转发表项的大小。

- 高可靠性

现场可插拔模块化电源，支持对芯片组的温度、电源与风扇的状态进行实时监控。

支持MLAG/LACP/VRRP/STP/RSTP/MSTP/Smart Link/BFD/ERPS等特性，全面且高效的保护网络中的数据流量。

- 三网合一服务支持与带宽保证的高品质应用

三网合一服务（IPTV，视频监控等）提供了高带宽。内置的QoS功能和灵活的队列技术可保障高品质的服务。

丰富的组播协议集（IGMP snooping/IGMP v1/v2/v3），支持2K组播组，每组4K逻辑复制，完美支持IPTV业务，有效控制组播时延。

- 全面的网络安全策略

支持用户级、交换机级和网络级的安全控制。

基本的IPv4 / IPv6 / MAC ACL可以分别过滤IPv4 / IPv6/报文，并且，额外提供扩展的IPv4 / IPv6 ACL可以在一个规则内同时匹配报文的二、三、四层信息。以上ACL均可以应用到物理端口、VLAN、端口组和VLAN组上，端口组或VLAN组内的成员共享一套ACL规则，可以节省TCAM资源。

ARP检测和IP源防护功能，防止网络遭受恶意ARP攻击。 提供CPU流量保护、风暴控制功能，优化CPU负载功能。集中式的802.1x认证，禁止非法用户访问网络。

- 出色的管理特性

支持丰富的管理接口，包括Console、带内网口和带外网口、USB口；支持SNMPv1/v2/v3，支持CLI命令行，Web网管，TELNET及FTP配置使设备管理更方便，并且支持SSH2.0、SSL等加密方式，使得管理更加安全。

- 产品规格

| 项目 | S9500-6QF-2HQF |
| --- | --- |
| 固定端口 | 6*40GE QSFP+端口， 2*100G QSFP28端口 |
| 管理端口 | 1个Console口、1个带外网管口、1个USB |
| 交换容量 | 3.6Tbps |
| 包转发率 | 1480Mpps |
| 工作温度 | -10~50°C |
| 存储温度 | -20~70°C |
| 工作湿度 | 10%～90%无凝结 |
| 存储湿度 | 5%～95%无凝结 |
| 外观尺寸 | 355(L)×215(W)×44(H)mm |
| 整机重量 | <3.3Kg |
| 防雷等级 | 6KV |
| 输入电压 | AC：90～264V/50～60Hz |
| 整机功耗 | <350W |

- 业务特性

| Feature |  |  | Description |
| --- | --- | --- | --- |
| Type | Sub Type | Feature |  |
| Ethernet basic features | Ethernet | interface | Ethernet interface operating modes(full duplex, half duplex, and auto) |
|  |  |  | negotiation) |
|  |  |  | Ethernet interface operating rates |
|  |  |  | Jumbo Frame |
|  |  |  | port-xconnect |
|  |  | Flow-control | Flow-control tx/rx |
|  |  | storm-control | Port based storm-control |
|  |  | Port-block | Port-block(know-unicast/unknow-unicast/know-multicast/unknow- |
|  |  |  | multicast/broadcast) |
|  |  | Port-isolate | L2/L3/All Port-isolate |
|  |  |  | Uni-direction isolate |
|  |  | L2 Protocol Tunnel | L2 Protocol Tunnel(support CDP/CFM/DOT1X/LLDP/SLOW-PROTO/STP/VTP |
|  |  | Forward mode | Store-and-forward |
|  |  |  | Cut-through |
|  | VLAN | VLAN Access mode | Access/Trunk |
|  |  |  | Default VLAN |
|  |  | VLAN Classification | VLAN Classification(port based/mac based/ip based/protocal based) |
|  |  | QinQ | Basic QinQ |
|  |  |  | Selective QinQ |
|  |  |  | VLAN Mapping（1:1 VLAN Translation） |
|  |  | VLAN Statistics | VLAN Statistics |
|  |  | Private VLAN | Private VLAN |
|  |  | Voice VLAN | Voice VLAN |
|  |  | Guest VLAN | Guest VLAN |
|  | MAC | MAC Address Table | Automatic learning and aging of MAC addresses |
|  |  |  | Hardware Learning |
|  |  |  | Static and dynamic MAC address entries |
|  |  |  | blackhole MAC |
|  |  | MAC Flapping detect | MAC Flapping detect |
|  |  | Port Bridge | Port Bridge |
|  | LAG | Link aggregation | Static-LAG & LACP |
|  |  |  | LAG load balance（SLB） |
|  |  |  | LAG load balance（DLB） |
|  |  |  | LAG load balance（RR） |
|  |  |  | LAG Self-healing |
|  |  |  | Link aggregation weighting |
| Ethernet Ring protection features | xSTP | STP | Spanning-Tree Protocol |
|  |  | RSTP | Rapid Spanning-Tree Protocol |
|  |  | MSTP | Multi-instance Spanning-Tree Protocol |
|  |  | Spanning-Tree Protocol Protection | BPDU Filter/Guard |
|  |  |  | Root Guard |
|  |  |  | Loop Guard |
|  |  |  | Anti TC-BPDU attack |
|  | ERPS | ERPS | Single ERPS ring |
|  |  |  | tangent ERPS rings |
|  |  |  | intersecting ERPS rings |
|  |  |  | compatible with RRPP |
|  | Loopback Detect | Loopback Detect | Loopback-detection |
| Layer2 Multicast | Layer2 Multicast | IGMP Snooping | IGMPv1/v2/v3 Snooping |
|  |  |  | Fast leave |
|  |  |  | Static IGMP snooping group |
|  |  | MVR | MVR（Multicast VLAN Registration） |
| IPv4 Forwarding | ARP | ARP | Static and dynamic ARP entries |
|  |  |  | Aging of ARP entries |
|  |  |  | Gratuitous ARP |
|  |  | ARP proxy | basic ARP-Proxy |
|  |  |  | local ARP-Proxy |
|  | IPv4 Unicast Routing | IPv4 Static Routes | IPv4 Static Routes |
|  |  |  | blackhole Routes |
|  |  |  | co-work with IP SLA |
|  |  |  | VRF（Virtual Routing and Forwarding） |
|  |  |  | uRPF check |
|  |  | RIP | RIPv1/v2 |
|  |  | Route policy | Route-map |
|  |  |  | IPv4 prefix-list |
|  |  | PBR | PBR（Policy-based Routing） |
|  |  | ICMP | ICMP redirect |
|  |  |  | ICMP unreachables |
|  |  | ECMP | ECMP(SLB) |
|  |  |  | ECMP(DLB) |
|  |  |  | ECMP(RR) |
|  |  |  | ECMP Self-healing |
|  |  | ip unnumbered | ip unnumbered |
|  | IPv4 Multicast Routing | IGMP | IGMPv1/v2/v3 |
|  |  |  | IGMP-Proxy |
|  |  |  | IGMP SSM Mapping |
|  |  | VRRP v3 | VRRP v3 |
|  | VRRP | VRRP | VRRP |
|  |  |  | Track for VRRP |
|  | Smart Link | Smart Link | multi-instance |
|  |  |  | load balance |
|  |  |  | Multi-Link |
|  |  |  | Monitor-link |
|  | MLAG | MLAG | MLAG basic |
|  |  |  | MLAG orphan Port |
| QoS features | QoS | Traffic classification | Traffic classification based on COS/DSCP (simple classification) |
|  |  |  | Traffic classification based on ACL ( complex classification) |
|  |  |  | Traffic classification based on inner header of the tunnel packets |
|  |  | Traffic behaviors | Queue scheduling |
|  |  |  | Remark the priority fields(COS/DSCP) of the packet based on ACL |
|  |  |  | Remark the priority fields(COS/DSCP) of the packet based on Table Map |
|  |  |  | Flow redirection |
|  |  |  | Flow mirror |
|  |  | Traffic policing | Traffic policing based on direction(in/out) of Port |
|  |  |  | Traffic policing based on direction(in/out) of VLAN |
|  |  |  | Traffic policing based on direction(in/out) of flow |
|  |  |  | Traffic policing based on direction(in/out) of aggregated flow |
|  |  | Traffic shaping | Queue based traffic shaping |
|  |  |  | Port based traffic shaping |
|  |  | Congestion management | SP（Strict Priority）scheduling |
|  |  |  | WDRR（Weighted Deficit Round Robin）scheduling |
|  |  |  | SP + WDRR mixed scheduling |
|  |  | Congestion avoidance | TD（Tail Drop） |
|  |  |  | WRED（Weighted Random Early Detection） |
|  |  | Traffic statistics | Packet counts and bytes statistics based on traffic classification |
|  |  |  | Packet counts and bytes statistics based on the color after traffic policing |
|  |  |  | Forwarded and discarded packet counts and bytes statistics |
| Data Center | VARP | Virtual gateway | VARP（Virtual-ARP） |
|  |  |  | VARP subnet |
| Security and management | System Security | SSH | SSHv1/v2 |
|  |  |  | RSA Key generation |
|  |  | RADIUS | RADIUS |
|  |  | TACAS+ | TACAS+ |
|  |  | AAA | Authentication |
|  |  |  | Authorization |
|  |  |  | Accounting |
|  |  | Dot1x | Port based dot1x |
|  |  |  | MAC based dot1x |
|  |  |  | Guest VLAN |
|  |  | ACL | MAC/IP ACL |
|  |  |  | Basic Mode ACL |
|  |  |  | Port-group ACL |
|  |  |  | VLAN-group ACL |
|  |  |  | IPv6 ACL |
|  |  |  | ACL UDF |
|  |  |  | Global ACL |
|  |  |  | Time Range |
|  |  | ARP Inspection | ARP Inspection |
|  |  | IP Source Guard | IP Source Guard |
|  |  | Port Security | Limitation on MAC address learning on interface |
|  |  | VLAN Security | Limitation on MAC address learning on VLAN |
|  |  | Control Plane Policy (COPP) | Black list/wihte list |
|  |  |  | Rate limit |
|  |  | CPU Traffic Limit | CPU Traffic Limit |
|  |  | Prevent DDOS attack | Prevent DDOS attack (ICMP Flood/Smurf/Fraggle/LAND/SYN Flood) |
|  |  | Login filter | Telnet/SSH ACL filtering |
|  |  |  | Telnet/SSH IPv6 ACL filtering |
|  |  | Link-Flapping detection | Link-Flapping detection |
|  | Network Management | DHCP | DHCP Server |
|  |  |  | DHCP Relay |
|  |  |  | DHCP Snooping |
|  |  |  | DHCP Client |
|  |  |  | DHCP Option82 |
|  |  |  | DHCP Option252 |
|  |  | RMON | RMON |
|  |  | sFlow | sFlow v4/v5 |
|  |  | IP SLA | IP SLA |
|  |  | EFD | Elephant Flow Detection |
|  |  | NTP | NTP（Network Time Protocol） |
|  |  | Errdisable | Errdisable detection and recovery |
|  |  | DNS | Static DNS Client |
|  |  | LLDP | LLDP |
| Management | Terminal Services | Command Line Interface | Configurations through CLI (Command Line Interface) |
|  |  | Help information | Banner configuration |
|  |  |  | Help information in English |
|  |  | Terminal service | Vty Terminal service |
|  |  |  | Console Terminal service |
|  | Management Port | Management interface | Inband management interface and configuration |
|  |  |  | Outband management interface and configuration |
|  |  | User privilege management | Privileged user proirity and privileged commands |
| Configuration and maintenance | Configuration Management | SNMP | Network management based on SNMPv1/v2c/v3 |
|  |  |  | Public and private MIB |
|  |  |  | Public and private Trap |
|  |  | WEB | Configuration and management based on WEB UI |
|  |  | RPC-API | Configuration and management based on RPC-API |
|  |  | SmartConfig | SmartConfig（Automatically configuration when system start） |
|  |  | OVSDB | Configuration and management based on OVSDB |
|  |  | system profile configuration | change the system specifications by choose different STM Profiles |
|  |  | License control | Feature configuration based on License |
|  |  | Restore factory default | Restore factory default configuration |
|  |  | configuration |  |
|  | File System | File system | File system(support directory and file management) |
|  |  | Upload and download | Upload and download files through FTP or TFTP |
|  |  |  | Upload and download files through Xmodem |
|  | Debugging And Maintenance | Debug | per-module Debug features |
|  |  |  | ICMP Debug |
|  |  | BHM | Software process monitor: BHM（Beat Heart Monitor） |
|  |  |  | Hardware Watch Dog |
|  |  | Log & alarm | CPU usage display and alarm |
|  |  |  | Memory usage display and alarm |
|  |  |  | Device temperature、PSU、FAN、status display and alarm |
|  |  |  | User operation logs |
|  |  |  | Management of logs, alarms, and debugging information |
|  |  | VCT | VCT（Virtual Cable Test） |
|  |  | system diagnostics | Detailed Diagnostic-information collection |
|  |  | Reboot | Manual reboot |
|  |  |  | Schedule Reboot |
|  |  |  | Reboot Information logging |
|  |  | network diagnostics | Ping |
|  |  |  | IPv6 Ping |
|  |  |  | Traceroute |
|  |  | mirror | Port mirror |
|  |  |  | Flow mirror |
|  |  |  | Remote mirror |
|  |  |  | Multi-destination mirror（m:n） |
|  |  |  | Use CPU as mirror source |
|  |  |  | Use CPU as mirror destination and analyze packet |
|  |  |  | ERSPAN |
|  |  | CPU statistics | To CPU/From CPU packets statistics |
|  |  | L2 Ping | layer2 network connectivity detection - L2Ping (MAC Ping/Trace) |
|  |  | UDLD | UDLD（Unidirectional Link Detection） |
|  |  | unidirectional | unidirectional forwarding of the fiber |
|  |  | Loopback | port loopback |
|  |  |  | hardware loopback（internal/external） |
|  |  | System time | Time configuration |
|  |  |  | Timezone |

- 订购信息

| 设备名称 | 设备信息 |
| --- | --- |
| S9500-6QF-2HQF | 6个40G QSFP端口，2个100G QSFP28端口； |
| 模块 | 模块信息 |
| QSFP+-40G-SR4-MM850 | 40G QSFP模块 SR4多模模块，MPO接口，波长850nm，传输距离100m |
| QSFP28-100G-SR4-MM850 | 100G QSFP28模块 SR4多模双芯，MPO接口，波长850nm，传输距离100m |

- 联系我们

|  | 深圳市万网博通科技有限公司 电话：0755-61133269 400 ：400-088-7500 网址：http://www.tg-net.cn 公司地址：深圳市龙华区大浪街道新石社区浪荣路17号3层 |
| --- | --- |

关于文档

本文档中的商标、图片、标识均归深圳市万网博通科技有限公司所有。

本文档可能含有预测信息，因此本文档信息仅供参考，不构成任何要约或承诺，本公司做出更正或修改恕不另行通知。

版权所有 © 深圳市万网博通科技有限公司  保留一切权利

V202309